1 Produktbeschreibung

2 Zugehöriges Material

# Produktbeschreibung

Hohe CO₂-Genauigkeit bei umfassender Kompatibilität mit Gebäudestandards

Der SCD43 bietet eine branchenführende Genauigkeit von ±(30 ppm + 3 % des Messwerts) sowie Kompatibilität mit den strengsten Gebäudestandards, darunter ANSI/ASHRAE 62.1-2022, California Title 24, RESET® Grade B und der WELL Building Standard (WELL v2). Darüber hinaus verfügt der SCD43 über einen erweiterten spezifizierten Messbereich von bis zu 5'000 ppm sowie einen Single-Shot-Betriebsmodus.

Merkmale:

- Photoakustische NDIR-Messtechnologie

- Kompakte Bauform von 10,1 × 10,1 × 6,5 mm³

- Vollständig gekapseltes Sensorgehäuse

- SMD-lötbar, Lieferung in Tape-and-Reel-Verpackung

- Integrierte Kompensation und Selbstkalibrierung

- Versorgungsspannungsbereich von 2,4–5,5 V

- Verschiedene Betriebsmodi und stromsparende Optionen

- Digitale I²C-Schnittstelle

Vorteile:

- Hohe Genauigkeit und selektive CO₂-Messung

- Platzsparende Integration

- Lange Lebensdauer von über 10 Jahren

- Robust gegenüber Staub, Feuchtigkeit und mechanischer Belastung

- Ermöglicht hochautomatisierte, kosteneffiziente Onboard-Montage

- Kalibrierter und linearisierter Sensorausgang mit langfristiger Driftkompensation (ASC)

- Einsetzbar in einer Vielzahl von Anwendungen

- Flexibles Verhältnis zwischen Stromverbrauch und Leistung

- Einfaches Design-in und unkomplizierte Schnittstellenanbindung

Anwendungen:

- Gewerbliche und private HLK-Systeme (Heizung, Lüftung, Klima)

- Wandthermostate

- Luftreiniger

- Luftqualitätsmonitore für Innenräume

Sensor Evaluation SEK-SCD43:

Das SEK-SCD43, bei dem ein SCD43 auf einem Entwicklungsboard integriert ist, ermöglicht das schnelle Testen des SCD43 für verschiedene Anwendungen.

Enthält:

- 1x SCD43 Entwicklungsboard

- 1x Adapterkabel

- 1x Jumperdraht-Set

1x SEK-SensorBridge erforderlich

# Zugehöriges Material

- Product-Page SCD43: https://sensirion.com/products/catalog/SCD43

- Picture SCD43 (no login required): https://brand.sensirion.com/share/yMZ47cfF3eM2AZPARwZ3/assets/57283

- Product-Page SEK-SCD43 : SEK-SCD43 - Evaluation kit for SCD43 CO2 sensors

- Datasheet: https://sensirion.com/resource/datasheet/scd4x ; https://sensirion.com/products/catalog/SEK-SCD43

- Indoor air quality brochure: https://sensirion.com/resource/flyer/iaq/brochure

- Carbon dioxide product flyer: https://sensirion.com/resource/flyer-co2

Guides:

- Design-in Guide: https://sensirion.com/resource/user_guide/scd4x/design-in

- Handling Instruction: https://sensirion.com/resource/user_guide/scd4x/handling_instructions

- Testing Guide: https://sensirion.com/resource/user_guide/scd4x/testing_guideline

Software:

- Embedded Driver: https://sensirion.com/resource/software/embedded/scd4x

- Arduino Driver: https://sensirion.com/resource/software/arduino/scd4x

- Raspberry Pi driver: https://sensirion.com/resource/software/raspberry-pi/scd4x

- Makecode for micro:bit: https://sensirion.com/resource/software/makecode/microbit/scd4x

- Arduino snippets SCD4x: https://sensirion.com/resource/software/arduino/scd4x/snippets

- BLE gadget SCD4x: https://sensirion.com/resource/software/ble-gadget/scd4x

- Python driver SCD4x: https://sensirion.com/resource/software/python/scd4x

- SEK-ControlCenter (all devices): https://sensirion.com/resource/viewer_software/controlcenter

Drawings, application notes & Zertifikate:

- REACH/ RoHS Certificate: https://sensirion.com/resource/certificate/scd/reach_rohs

- Step File: https://sensirion.com/resource/cad/scd4x ; https://sensirion.com/resource/cad/sek-scd41

- Low power operation SCD4x: https://sensirion.com/resource/application_note/scd4x/low_power_operation

- What type of NDIR sensors exist and how do they work? https://sensirion.com/resource/application_note/ndir-sensors-types

- Altium footprint SCD4x: https://sensirion.com/resource/software/altium_footprint/scd4x

- Calibration certification SCD4x : https://sensirion.com/resource/certificate/calibration-certification-scd4x